RA2311026011010
Siddhesh Khankhoje

Hybrid and Electric Vehicles

CLA-1 Assesment

Problem Statement:

Modern gasoline engines use three-way catalytic converters (TWC) to control exhaust emissions.
The Oxygen Storage Capacity (OSC) of the catalytic converter plays a critical role in maintaining stoichiometric air–fuel ratio by absorbing and releasing oxygen during transient engine operation.

The OSC of the oxidation catalyst depends on:

- Catalyst volume (L)

- Washcoat loading (g/L)

- CeO₂ weight fraction in the washcoat

- Utilisation efficiency (η)

The objective of this work is to:

- Develop a physics-based OSC model

- Generate lookup tables (LUTs) for OSC

- Visualize the sensitivity of OSC using:

  - Volume vs Washcoat loading

  - Volume vs CeO₂ wt. fraction

  - Volume vs Utilisation efficiency

- Create OSC contour plots and a 3D surface plot for calibration and injector-control applications.

Solution:

- Lookup table:

- Volume v/s Wash coat loading

- Volume v/s Ceria (CeO2) %weight fraction

- Volume v/s Utilisation efficiency

- 3D Surface Plot: OSC

- Program (Python code):

- import numpy as np

- import pandas as pd

- import matplotlib.pyplot as plt

- from mpl_toolkits.mplot3d import Axes3D

- # ===============================

- # CONSTANTS

- # ===============================

- CPSI = 750

- k_factor = 5.5

- O2_PER_G_CEO2 = 0.11

- # Nominal values

- washcoat_nom = 160        # g/L

- ceo2_nom = 0.275          # wt fraction

- eta_nom = 0.70

- # ===============================

- # AXES RANGES

- # ===============================

- volume = np.linspace(0.86, 1.0, 60)

- washcoat = np.linspace(120, 200, 60)

- ceo2_frac = np.linspace(0.20, 0.35, 60)

- eta = np.linspace(0.65, 0.75, 60)

- # ===============================

- # OSC FUNCTION

- # ===============================

- def compute_osc(volume, washcoat, ceo2_frac, eta):

- washcoat_mass = volume * washcoat

- ceo2_mass = washcoat_mass * ceo2_frac

- stored_o2 = ceo2_mass * O2_PER_G_CEO2 * eta

- osc_geom = CPSI * k_factor * volume

- return osc_geom * stored_o2

- # ===============================

- # 1. Volume vs Washcoat Loading

- # ===============================

- V1, W = np.meshgrid(volume, washcoat)

- OSC_vw = compute_osc(V1, W, ceo2_nom, eta_nom)

- plt.figure()

- c1 = plt.contour(V1, W, OSC_vw)

- plt.clabel(c1)

- plt.xlabel("Catalyst Volume (L)")

- plt.ylabel("Washcoat Loading (g/L)")

- plt.title("OSC Contour: Volume vs Washcoat Loading")

- plt.show()

- # ===============================

- # 2. Volume vs CeO2 wt fraction

- # ===============================

- V2, C = np.meshgrid(volume, ceo2_frac)

- OSC_vc = compute_osc(V2, washcoat_nom, C, eta_nom)

- plt.figure()

- c2 = plt.contour(V2, C, OSC_vc)

- plt.clabel(c2)

- plt.xlabel("Catalyst Volume (L)")

- plt.ylabel("CeO₂ Weight Fraction")

- plt.title("OSC Contour: Volume vs CeO₂ Fraction")

- plt.show()

- # ===============================

- # 3. Volume vs Utilisation Efficiency

- # ===============================

- V3, E = np.meshgrid(volume, eta)

- OSC_ve = compute_osc(V3, washcoat_nom, ceo2_nom, E)

- plt.figure()

- c3 = plt.contour(V3, E, OSC_ve)

- plt.clabel(c3)

- plt.xlabel("Catalyst Volume (L)")

- plt.ylabel("Utilisation Efficiency (η)")

- plt.title("OSC Contour: Volume vs Utilisation Efficiency")

- plt.show()

- # ===============================

- # 4. FILLED OSC CONTOUR

- # ===============================

- plt.figure()

- plt.contourf(V1, W, OSC_vw)

- plt.colorbar(label="OSC")

- plt.xlabel("Catalyst Volume (L)")

- plt.ylabel("Washcoat Loading (g/L)")

- plt.title("Filled OSC Contour: Volume vs Washcoat")

- plt.show()

- # ===============================

- # 5. 3D SURFACE PLOT

- # ===============================

- fig = plt.figure()

- ax = fig.add_subplot(111, projection="3d")

- ax.plot_surface(V1, W, OSC_vw)

- ax.set_xlabel("Catalyst Volume (L)")

- ax.set_ylabel("Washcoat Loading (g/L)")

- ax.set_zlabel("OSC")

- ax.set_title("3D Surface Plot: OSC")

- plt.show()

- # ===============================

- # 6. LOOKUP TABLE (LUT)

- # ===============================

- volume_lut = np.linspace(0.86, 1.0, 10)

- washcoat_lut = np.linspace(120, 200, 10)

- ceo2_lut = np.linspace(0.20, 0.35, 10)

- eta_lut = np.linspace(0.65, 0.75, 10)

- osc_lut = compute_osc(volume_lut, washcoat_lut, ceo2_lut, eta_lut)

- lut = pd.DataFrame({

- "Volume_L": volume_lut,

- "Washcoat_g_per_L": washcoat_lut,

- "CeO2_fraction": ceo2_lut,

- "Efficiency_eta": eta_lut,

- "OSC": osc_lut

- })

- print("\n=== OSC LOOKUP TABLE ===\n")

- print(lut)

- lut.to_csv("osc_lookup_table.csv", index=False)